AeroCardia

Health Scores Testing Protocol

Optimised Protocol Reference  ·  v1.0

| SHORT VERSION ~15 minutes 5 tests · 3 domains | STANDARD VERSION ~20 minutes 6 tests · 3 domains | EXTENDED VERSION ~30 minutes 8 tests · 3 domains |
| --- | --- | --- |

This document specifies three versions of the AeroCardia Health Scores battery. All versions produce the same three domain scores (Pulmonary, Cardiovascular, Functional Mobility) and feed the same normative engine. Version selection is driven by the user's available time, age, and condition profile.

# 1. Protocol Design Rationale

Each version was designed around a single principle: maximize clinically meaningful signal per minute of user time. Tests were selected and ordered based on three criteria:

- Signal density — how many scored metrics does the test yield per minute?

- Normative strength — how robust is the reference dataset backing the score?

- Saturation range — does the test discriminate across the full ability spectrum, or does it floor/ceiling in important user segments?

The table below shows which tests appear in each version and why.

| Test | Full (20 min) | Short (15 min) | Extended (30 min) | Primary Metrics |
| --- | --- | --- | --- | --- |
| Resting Baseline | ✓ | ✓ | ✓ | HR, SpO₂, HRV, I:E ratio |
| Breath-Hold | ✓ | — | ✓ | CO₂ tolerance, BOLT score |
| Spirometry FEV₁/FVC | ✓ | ✓ | ✓ | FEV₁, FVC, FEV₁/FVC |
| YMCA Step Test | ✓ | ✓ | ✓ | HR recovery, VO₂ proxy |
| Single-Leg Balance | ✓ | ✓ | ✓* | Balance time, sway |
| Sit-to-Stand / Squat | ✓ | ✓ | ✓* | Reps, fatigue index |
| Front Plank | — | — | ✓ | Core endurance (seconds) |
| Wall Sit | — | — | ✓ | Isometric quad endurance |

* Age-appropriate variant selected automatically by the app (eyes-open only vs. composite; STS vs. squat vs. wall sit).

# 2. Pre-Test Requirements

Apply to all three versions. Administer before any test begins.

| Equipment Required - AeroCardia device + AeroStim app - Chair without armrests (against wall) - 30 cm step box (Tests 3 / 4) - Nose clip (spirometry) - Flat, non-slip indoor surface - 10 m course (30-min version 6MWT only) | Participant Instructions - Do not test if sick, injured, or medicated for acute conditions - No smoking, exercise, caffeine, or alcohol ≥ 30 min prior - Use washroom before beginning - Sit and rest quietly for 5 minutes before Test 1 |
| --- | --- |

SHORT VERSION · ~15 MINUTES Maximum signal in minimum time. All three domains scored. Ideal for high-volume screening, busy users, and initial onboarding.

## Design Decisions — 15-Minute Version

The 15-minute protocol is built around the three tests with the best signal-to-time ratio in the battery:

- Breath-hold is removed — it contributes only 10% of the Pulmonary Score and has the thinnest normative base (n=25). Its absence reduces protocol time by ~3 minutes with minimal scoring impact.

- Spirometry is shortened to 2 trials (vs. 3). The best of two efforts captures >95% of the true FEV₁/FVC in most users. The third trial primarily benefits borderline cases and can be added ad hoc.

- Resting baseline is trimmed to 3 minutes (quiet breathing only; the controlled deep-breathing minute is removed). This still captures resting HR, SpO₂, HRV, and I:E ratio — the four highest-value passive metrics.

- Balance uses the age-appropriate variant with no rest between legs (30 s rest between R and L is preserved).

- STS / Squat proceeds immediately after balance with no inter-test gap.

## Time Budget — 15-Minute Version

| Step | Time |
| --- | --- |
| Pre-test rest | 5 min |
| Test 1: Resting Baseline | 3 min |
| Inter-test rest | 1 min |
| Test 2: Spirometry (2 trials) | 3 min |
| Inter-test rest | 1 min |
| Test 3: YMCA Step Test | 6 min |
| Test 4: Balance (eyes-open) | 2 min |
| Test 5: STS / Squat | 2 min |
| TOTAL | ~17 min |

Test 1 of 5 — Resting Baseline

| TEST 1 Resting Baseline | PULMONARY SCORE ⏱ 3 min |
| --- | --- |

| Purpose Capture resting HR, SpO₂, HRV, and I:E ratio. Establishes the cardiopulmonary baseline against which exercise responses are compared. | Setup Seated, back supported, feet flat. AeroCardia device in mouth. Quiet room. |
| --- | --- |

Procedure

- Sit quietly, breathe normally into the device. Avoid talking or moving.

- Maintain quiet baseline breathing for 3 minutes. The app will signal when complete.

- Rest for 1 minute before Test 2.

Key Metrics Captured

| → Resting HR (bpm) | → SpO₂ (%) |
| --- | --- |
| → HRV — RMSSD (ms) | → I:E Ratio |
| → Tidal volume (L) | → Breathing rate (br/min) |

| APP STATE FLAGS Test 1 · Resting Baseline |  |  |
| --- | --- | --- |
| Phase | State Code Emitted | What it marks / what to extract |
| Pre-rest | T1.PREREST | Session opens. 5-min pre-test rest begins. All streams idle — no recording. |
| Quiet start | T1.QUIET.START | Device in mouth. Streams active: HR, SpO2, O2, CO2, pressure, accel, tilt. Extract: resting HR, SpO2, HRV/RMSSD, RSA amplitude, I:E ratio. |
| Quiet end | T1.QUIET.END | 2-min quiet window closes. Freeze all resting baseline values. |
| Deep-breath start | T1.DEEP.START | Controlled deep breathing begins (max exhale -> max inhale). Extract: tidal volume, pressure amplitude, breath consistency. |
| Deep-breath end | T1.DEEP.END | 1-min deep-breathing window closes. I:E ratio and RRV metrics locked. |
| Test complete | T1.COMPLETE | Device removed. 1-min inter-test rest begins. Streams idle. |
| Rest end | T1.REST.END | Inter-test rest complete. Prompt: begin Test 2. |

Test 2 of 5 — Spirometry

| TEST 2 Spirometry (FEV₁ / FVC) | PULMONARY SCORE ⏱ 3 min |
| --- | --- |

| Purpose Measure forced expiratory volumes. FEV₁/FVC carries 30% of Pulmonary Score weight — the single largest contributor. | Setup Seated with nose clip. AeroCardia in mouth. Upright posture, feet flat. |
| --- | --- |

Procedure

- Take a full, maximal breath in to total lung capacity.

- Blast air out as hard and fast as possible until lungs are completely empty, then inhale normally.

- Breathe normally for 60 seconds.

- Repeat steps 1–3 once more (2 trials total). Best effort is recorded.

- Rest for 1 minute before Test 3.

Key Metrics Captured

| → FEV₁ (L) | → FVC (L) |
| --- | --- |
| → FEV₁/FVC ratio | → FEV₀.₅ (L) |
| → Peak expiratory pressure | → Inspiratory flow rate (PIF) |

| APP STATE FLAGS Test 2 · Spirometry |  |  |
| --- | --- | --- |
| Phase | State Code Emitted | What it marks / what to extract |
| Trial start | T2.TRIAL.START[n] | Trial n begins (n=1,2,3). Device in, nose clip on. FEV1/FVC extraction window opens. |
| Peak flow | T2.PEAK[n] | Peak expiratory flow detected in trial n. Timestamp for flow-volume curve alignment. |
| Exhale end | T2.EXHALE.END[n] | Full exhalation confirmed (flow < 0.025 L/s sustained >1 s). FEV0.5, FEV1, FVC for this trial locked. |
| Inter-trial rest | T2.INTERTRIAL[n] | Recovery breathing begins. 60-s inter-trial rest. Running best-of-n updated. |
| Best locked | T2.BEST.LOCKED | All trials complete. Best FEV1 and FVC locked (may differ by trial, per ATS/ERS). FEV1/FVC ratio computed. |
| Test complete | T2.COMPLETE | Device removed. 1-min inter-test rest begins. |
| Rest end | T2.REST.END | Ready for next test. |

Test 3 of 5 — YMCA 3-Minute Step Test

| TEST 3 YMCA 3-Minute Step Test | CARDIOVASCULAR SCORE ⏱ 6 min |
| --- | --- |

| Purpose Submaximal aerobic challenge. 1-minute post-exercise HR recovery is the primary cardiovascular metric, with VO₂max estimated as a display-only output. | Setup Standing in front of 30 cm step box. AeroCardia device in mouth throughout. |
| --- | --- |

Procedure

- Stand up in front of the box and wait for 10 seconds.

- Step up and down on the box at the cadence set by the app (96 bpm). Maintain full upright posture at the top of each step.

- Continue for 3 minutes. Stop immediately if dizzy, overly breathless, or feel unsteady.

- Sit down immediately when the 3-minute timer ends. Remain seated and still.

- The app will record your 1-minute recovery HR automatically.

- Remain seated for 2 minutes before Test 4.

Key Metrics Captured

| → 1-min recovery HR (bpm) | → VO₂max estimate (mL/kg/min) |
| --- | --- |
| → Step cadence symmetry | → SpO₂ delta (vs. rest) |
| → RER during exercise | → VE/VCO₂ slope |

| APP STATE FLAGS Test 3 · YMCA 3-Minute Step Test |  |  |
| --- | --- | --- |
| Phase | State Code Emitted | What it marks / what to extract |
| Standing baseline | T3.STANDING | Participant stands. 10-s HR + SpO2 sample. Orthostatic HR delta vs. T1 seated baseline computed here. |
| Step start | T3.STEP.START | Stepping begins at 96 bpm. All streams active. VE/VCO2, O2 pulse proxy, SpO2, cadence symmetry windows open. |
| Min 1 | T3.MIN1 | 60-s snapshot: HR + SpO2 logged. |
| Min 2 | T3.MIN2 | 120-s snapshot: HR + SpO2 logged. |
| Step end | T3.STEP.END | 3-min stepping complete. Participant sits. Recovery HR clock starts. |
| Recovery 1 min | T3.REC.1MIN | 1-min recovery HR captured. Primary YMCA scoring input. VO2max regression locked. |
| Recovery 2 min | T3.REC.2MIN | 2-min recovery HR captured. Secondary flag: <22 bpm drop check. |
| SpO2 delta | T3.SPO2.DELTA | SpO2 nadir during stepping vs. T1 baseline computed. Exercise desaturation flag evaluated (>=3% drop). |
| Vent settled | T3.VENT.SETTLED | Breathing rate returns to <20 br/min. Ventilatory recovery rate locked. |
| Test complete | T3.COMPLETE | 2-min inter-test rest begins. |
| Rest end | T3.REST.END | Ready for balance test. |

Test 4 of 5 — Single-Leg Balance

| TEST 4 Single-Leg Balance | FUNCTIONAL MOBILITY ⏱ 2–4 min |
| --- | --- |

| Purpose Static postural control, core stability, and ankle proprioception. Protocol variant selected automatically based on age and condition profile. | Setup Chair nearby for safety. AeroCardia not required. Flat, non-slip surface. |
| --- | --- |

Procedure

- Stand on right leg, left thigh raised parallel to the floor, hands at side. Hold as long as possible, up to 60 seconds.

- Rest 30 seconds.

- Repeat on left leg.

- AGE ≤ 45, no limiting condition: repeat steps 1–3 with eyes closed.

- Rest 1 minute before Test 5.

Key Metrics Captured

| → Hold time R + L (seconds) | → Sway path length (cm) |
| --- | --- |
| → Sway frequency decomposition | → Balance composite score (0–100) |

| APP STATE FLAGS Test 4 · Single-Leg Balance |  |  |
| --- | --- | --- |
| Phase | State Code Emitted | What it marks / what to extract |
| Right start (EO) | T4.R.EO.START | Right-leg eyes-open hold begins. Accel sway path + frequency tracking active. Hold clock starts. |
| Right end (EO) | T4.R.EO.END | Right-leg hold complete (step-off or 60-s cap). Duration locked. Sway path + freq decomposition computed. |
| Inter-leg rest | T4.INTER.REST | 30-s rest between legs. |
| Left start (EO) | T4.L.EO.START | Left-leg eyes-open hold begins. |
| Left end (EO) | T4.L.EO.END | Left-leg hold complete. EO average (R+L)/2 and L-R asymmetry index computed. |
| EC right start | T4.R.EC.START | [Age <=45 only] Right-leg eyes-closed hold begins. Separate extraction window. |
| EC right end | T4.R.EC.END | Eyes-closed right hold complete. |
| EC inter rest | T4.EC.INTER.REST | 30-s rest. |
| EC left start | T4.L.EC.START | Left-leg eyes-closed hold begins. |
| EC left end | T4.L.EC.END | Complete. Composite score (EO avg + EC avg) locked. |
| Test complete | T4.COMPLETE | 1-min inter-test rest begins. |
| Rest end | T4.REST.END | Ready for strength test. |

Test 5 of 5 — Lower-Body Strength

| TEST 5 Lower-Body Strength / Endurance | FUNCTIONAL MOBILITY ⏱ 2 min |
| --- | --- |

| Purpose Lower-body muscular endurance and functional fall risk. Protocol variant routed by app based on age and condition profile. | Setup Chair against wall (no armrests). AeroCardia not required. Arms crossed on chest throughout. |
| --- | --- |

Procedure

- AGE > 45 / limiting condition — Sit-to-Stand: Rise to full stand and sit back down as many times as possible in 30 seconds. Keep arms crossed on chest.

- AGE ≤ 45, no limiting condition — Squats: Perform as many squats as possible with full depth (thighs parallel to floor) and straight spine. Stop when unable to maintain form. Test auto-ends at 2 minutes.

- The app counts reps automatically via the accelerometer.

Key Metrics Captured

| → Rep count | → Max acceleration per rep |
| --- | --- |
| → Delta time between reps (fatigue rate) | → Eccentric:Concentric ratio (squat only) |

EXTENDED VERSION · ~30 MINUTES Full battery plus two new constructs: core endurance and isometric quad endurance. Designed for physio, occupational health, and performance contexts.

## Design Decisions — 30-Minute Version

The 30-minute protocol adds back everything removed from the 15-minute version, restores the full 3-trial spirometry and 4-minute resting baseline, and introduces two new tests targeting constructs absent from the current battery:

- Breath-hold is restored — the CO₂ tolerance and BOLT-equivalent score provide ventilatory control data relevant for respiratory rehab, occupational health, and users with anxiety or dysfunctional breathing patterns.

- Front Plank (timed, 3-min cap) — core endurance is independently predictive of low back pain and injury risk. No current test in the battery addresses the trunk stability construct. Added after the STS/squat to keep lower-limb fatigue from contaminating the plank.

- Wall Sit (timed isometric, 2-min cap) — adds isometric quadriceps endurance, a distinct construct from both the dynamic STS and squat tests. Accessible across a wide ability range (15 s for deconditioned older adults to 90+ s for fit young adults). Your tilt sensor verifies knee angle continuously.

- The 6MWT remains the cardiac-condition alternative to the YMCA step test and is not run concurrently with it.

## Time Budget — 30-Minute Version

| Step | Time |
| --- | --- |
| Pre-test rest | 5 min |
| Test 1: Resting Baseline | 4 min |
| Inter-test rest | 1 min |
| Test 2: Breath-Hold | 3 min |
| Inter-test rest | 1 min |
| Test 3: Spirometry (3 trials) | 4 min |
| Inter-test rest | 1 min |
| Test 4: YMCA Step Test | 6 min |
| Test 5: Balance | 2–4 min |
| Inter-test rest | 1 min |
| Test 6: STS / Squat | 2 min |
| Test 7: Front Plank | 3 min |
| Inter-test rest | 1 min |
| Test 8: Wall Sit | 3 min |
| TOTAL | ~30 min |

Tests 1–6

Tests 1 through 6 follow the same procedures as the Standard (20-minute) protocol. See the original AeroCardia Health Scores Testing Protocol document for full step-by-step instructions for:

- Test 1: Resting Baseline (4 min)

- Test 2: Breath-Hold & Ventilatory Control (3 min)

- Test 3: Spirometry FEV₁/FVC — 3 trials (4 min)

- Test 4: YMCA 3-Minute Step Test (6 min)

- Test 5: Single-Leg Balance (2–4 min, age-routed)

- Test 6: 30-Second Sit-to-Stand / Squat (2 min, age-routed)

Inter-test rest before new tests

Allow 1 minute of quiet seated rest after Test 6 before beginning the plank.

The plank and wall sit are low-cardiovascular-demand tests and do not require extended recovery between them.

| APP STATE FLAGS Test 5 · Lower-Body Strength (STS or Squat) |  |  |
| --- | --- | --- |
| Phase | State Code Emitted | What it marks / what to extract |
| Test start | T5.START | Clock starts. Accel + tilt rep detection active. STS: 30-s cap. Squat: 2-min cap. |
| Rep confirmed | T5.REP[n] | Rep n confirmed: full stand+sit (STS) or squat depth verified (tilt >=70 deg). Timestamp for delta-time fatigue analysis. |
| Depth fail | T5.DEPTHFAIL[n] | [Squat only] Rep n rejected — tilt <70 deg. Excluded from valid count. Logged for form-break analysis. |
| Fatigue window | T5.FATIGUE.OPEN | Final third opens (20 s rem for STS; 40 s rem for squat). Fatigue rate comparison window starts. |
| Test end | T5.END | Time cap reached or stop. Valid rep count, fatigue index, and E:C ratio (squat) locked. |
| Test complete | T5.COMPLETE | Standard/15-min: session ends, generate report. 30-min: 1-min rest begins. |
| Rest end | T5.REST.END | [30-min only] Ready for Front Plank. |

Test 7 of 8 — Front Plank

| TEST 7 Front Plank Hold | FUNCTIONAL MOBILITY ⏱ 3 min |
| --- | --- |

| Purpose Core muscular endurance. Independently predictive of low back pain risk and functional longevity. First test in the battery to address trunk stability directly. | Setup Flat non-slip floor. Forearms and toes only. AeroCardia not required. Timer visible to participant. |
| --- | --- |

Procedure

- Position yourself in a forearm plank: elbows directly under shoulders, body in a straight line from head to heels, toes on floor.

- Hold the position as long as possible. Maintain a neutral spine — no hip sag, no raised hips.

- Stop when you can no longer maintain form, or at the 3-minute cap. The app will detect hip sag via the accelerometer if the device is worn on the body.

- Rest for 1 minute before Test 8.

Key Metrics Captured

| → Hold time (seconds) | → Form-break detection (hip sag angle) |
| --- | --- |
| → Max tilt deviation during hold |  |

| APP STATE FLAGS Test 7 · Front Plank |  |  |
| --- | --- | --- |
| Phase | State Code Emitted | What it marks / what to extract |
| Position check | T7.POSCHECK | Starting tilt angle recorded. Hip and shoulder alignment verified. Baseline angle locked. |
| Hold start | T7.HOLD.START | Hold clock starts. Hip-sag monitoring active (tilt delta vs. baseline). |
| Sag warning | T7.SAG.WARN | Hip tilt deviates >10 deg from baseline. Form correction prompt issued. Sag onset timestamp logged. |
| Sag fail | T7.SAG.FAIL | Deviation persists >3 s after warning. Hold time locked at this point as the valid score. |
| Cap reached | T7.CAP | 3-min cap reached without form failure. Duration = 180 s. |
| Hold end | T7.HOLD.END | Plank ends (voluntary stop, sag fail, or cap). Duration locked. Percentile computed. |
| Test complete | T7.COMPLETE | 1-min inter-test rest begins. |
| Rest end | T7.REST.END | Ready for Wall Sit. |

Test 8 of 8 — Wall Sit

| TEST 8 Wall Sit — Isometric Hold | FUNCTIONAL MOBILITY ⏱ 3 min |
| --- | --- |

| Purpose Isometric quadriceps endurance. Distinct from dynamic STS and squat tests. Accessible across a wide ability range. Also has secondary cardiovascular relevance — isometric wall sits acutely reduce resting blood pressure. | Setup Flat wall, no chair. Feet shoulder-width apart, 30–45 cm from wall. AeroCardia not required. |
| --- | --- |

Procedure

- Place your back flat against the wall and slide down until hips and knees are at 90°. The app will verify the knee angle via tilt sensor.

- Hold as long as possible, up to 2 minutes. Keep arms loose at sides or crossed on chest — do not press on thighs.

- Stop when you can no longer maintain the 90° position. The tilt sensor will flag form breakdown automatically.

Key Metrics Captured

| → Hold time (seconds) | → Knee angle (degrees, via tilt) |
| --- | --- |
| → HR during hold | → Form-break timestamp |

# 5. Protocol Routing by User Profile

The app selects the appropriate test variant before the session begins, based on the user's stored profile. The normative engine is stateless — routing happens in the application layer.

| User Profile | Balance Protocol | Strength Protocol |
| --- | --- | --- |
| Age ≤ 45, no conditions | eyes-open + eyes-closed composite | Bodyweight squats to exhaustion (2-min cap) |
| Age > 45, or any limiting condition | Eyes-open only (avg R + L, 60 s cap) | 30-second sit-to-stand |
| Cardiac condition present | Eyes-open only | 30-second sit-to-stand |
| Knee / lower-limb injury | Eyes-open only | Wall sit (isometric, if tolerated) |

Cardiac Condition Routing

Users with a cardiac condition flag skip the YMCA Step Test entirely.

They are routed to the 6-Minute Walk Test (6MWT) instead, scored against Enright & Sherrill 1998 predicted distance.

UI label changes to 'Functional Exercise Capacity' rather than 'Cardiovascular Score'.

6MWT is self-paced and submaximal — validated for use in stable cardiac rehabilitation.

# 6. Score Outputs by Version

All three versions produce the same three domain scores fed into the same normative engine. The number of input metrics differs, not the scoring structure.

| Domain Score | 15-Min Inputs | 30-Min Additional Inputs |
| --- | --- | --- |
| Pulmonary Score | Resting HR · SpO₂ flag · FEV₁ · FVC · FEV₁/FVC | + Breath-hold percentile (10% weight) |
| Cardiovascular Score | YMCA 1-min recovery HR · VO₂max (display only) | No change (same inputs) |
| Functional Mobility Score | Balance composite · STS reps or squat reps | + Plank hold time · Wall sit hold time (scored, equal weight added to mobility sub-score) |

Implementation Note — Plank & Wall Sit Scoring

Normative data for the front plank and wall sit are sufficient to add to the Functional Mobility Score in the 30-minute version.

Recommended approach: add both as a third sub-component — Mobility Score = 33% balance + 33% STS/squat + 17% plank + 17% wall sit.

This weighting is provisional. Adjust once internal validation data is available from real-world sessions.

The normative spec (v2.3) will need a plank module (normative/plank.py) and a wall sit module (normative/wallsit.py) before the 30-min scoring pipeline is fully live.

* Note: The 90/90 lunge hold and other pure flexibility tests were evaluated and deferred. No sufficiently powered age-stratified normative database currently exists to support scored percentile output for these tests. They remain candidates for an exploratory/unscored data capture column in future versions.

Appendix A — Clinical Flags & Parameters of Special Importance

This appendix catalogues every threshold within the AeroCardia battery that triggers a clinical flag, warrants special interpretation, or has established prognostic significance in the literature. Flags do not constitute a diagnosis — they are signals that warrant follow-up, contextualisation against the user's full profile, or referral where appropriate. Each entry notes the specific threshold, the source or guideline that established it, and a priority level.

Priority Levels

⚠ High Priority — Threshold is established in clinical guidelines; warrants immediate attention or referral if persistent.

◆ Moderate — Threshold is research-derived and clinically meaningful; warrants contextualisation and monitoring.

● Informational — Useful interpretive signal; not independently actionable but informs the overall profile.

A1 — Cardiovascular Flags

| Parameter | Threshold / Flag Condition | Source / Guideline | Priority |
| --- | --- | --- | --- |
| Resting Heart Rate | < 50 bpm (bradycardia, non-athletic); > 100 bpm (tachycardia at rest) | ACC/AHA Heart Failure Guidelines; ESC 2021 | ⚠ High Priority |
| Resting Heart Rate — elevated | > 80 bpm at rest: associated with 45% higher all-cause mortality vs. < 60 bpm in general population | Aune et al. Eur J Prev Cardiol 2017 | ◆ Moderate |
| Heart Rate Change on Standing (Orthostatic HR Response) | Rise > 30 bpm within 10 min of standing suggests POTS. Rise > 20 bpm in 60–69-year-olds also clinically significant. Rise > 40 bpm in adolescents (ages 12–19) meets paediatric POTS criteria. | Sheldon et al. Heart Rhythm 2015 (POTS diagnostic criteria) | ⚠ High Priority |
| HR Recovery — 1 min post-YMCA Step Test | < 12 bpm drop from peak to 1-min recovery: independent predictor of mortality (HR 2.0× increased risk) | Cole et al. NEJM 1999; Nishime et al. JAMA 2000 | ⚠ High Priority |
| HR Recovery — 2 min post-exercise | < 22 bpm drop at 2 minutes: abnormal; associated with cardiovascular mortality | Shetler et al. J Am Coll Cardiol 2001 | ◆ Moderate |
| Heart Rate Variability — RMSSD | < 20 ms at rest: very low vagal tone, associated with autonomic dysfunction and elevated CVD risk | Task Force ESC/NASPE 1996; Shaffer & Ginsberg Front Public Health 2017 | ◆ Moderate |
| Heart Rate Variability — RMSSD | 20–50 ms: average healthy adult; > 50 ms: good autonomic health; > 100 ms: athletic | Shaffer & Ginsberg Front Public Health 2017 | ● Informational |
| SpO₂ — Resting | < 95%: clinically significant hypoxaemia; triggers Pulmonary Score cap at 40 in normative engine | O'Driscoll et al. Thorax 2017; BTS O₂ Guidelines | ⚠ High Priority |
| SpO₂ — Resting (COPD / Asthma threshold) | < 88%: abnormal in COPD or asthma at rest; these patients may have chronically suppressed hypoxic drive — a separate clinical threshold from the general population < 95% flag | O’Driscoll et al. Thorax 2017; BTS O₂ Guidelines; GOLD COPD 2023 | ⚠ High Priority |
| SpO₂ — Exercise Desaturation (Delta Test 1→4) | Drop ≥ 3% from resting baseline during YMCA step test: exercise-induced desaturation; flag for pulmonary vascular disease, ILD, severe COPD | ATS/ERS Statement on Exercise Testing 2003; Hadeli et al. Chest 2001 | ⚠ High Priority |
| SpO₂ — Absolute During Exercise | < 90% at any point during exercise: contraindication to continuing unsupervised exercise | ATS/ERS Exercise Testing Statement 2003 | ⚠ High Priority |
| HR–SpO₂ Divergence During Exercise | HR rising steeply while SpO₂ drops ≥ 3%: suggests impaired gas exchange (pulmonary limitation) rather than pure deconditioning | Wasserman et al. Principles of Exercise Testing, 5th ed. | ◆ Moderate |
| VE/VCO₂ Slope | > 34: elevated ventilatory inefficiency; > 40: severe, associated with poor prognosis in heart failure | Arena et al. Eur J Heart Fail 2007; Francis et al. Circulation 2000 | ⚠ High Priority |
| Cross-Test Resting HR Drift | > 10 bpm rise in resting HR from Test 1 to pre-Test 4: suggests incomplete recovery or autonomic reactivity to earlier tests | AeroCardia internal quality metric; Plews et al. IJSPP 2013 | ● Informational |

A2 — Pulmonary & Respiratory Flags

| Parameter | Threshold / Flag Condition | Source / Guideline | Priority |
| --- | --- | --- | --- |
| FEV₁/FVC Ratio | < 0.70 (post-bronchodilator): diagnostic criterion for obstructive airway disease (COPD/asthma) | GOLD Initiative COPD 2023; ATS/ERS Spirometry Standards 2005 | ⚠ High Priority |
| FEV₁/FVC Ratio — GLI Z-score | Z-score < −1.645 (below 5th percentile for age/sex/height/ethnicity): lower limit of normal | Quanjer et al. Eur Respir J 2012 (GLI-2012) | ⚠ High Priority |
| FVC — GLI Z-score | Z-score < −1.645: reduced FVC; may indicate restriction, air trapping, or submaximal effort | Quanjer et al. Eur Respir J 2012 | ⚠ High Priority |
| FEV₁ — GLI Z-score | Z-score < −1.645: reduced FEV₁; combined with normal FVC suggests obstruction | Quanjer et al. Eur Respir J 2012 | ⚠ High Priority |
| FEV₀.₅ | Disproportionately low relative to FEV₁: suggests large airway obstruction; more sensitive than FEV₁ alone for early obstruction | Pellegrino et al. Eur Respir J 2005 | ◆ Moderate |
| Resting Breathing Rate | > 25 breaths/min at rest (tachypnoea): independent predictor of cardiovascular events and hospitalisation | Fieselmann et al. J Gen Intern Med 1993; Cretikos et al. MJA 2008 | ⚠ High Priority |
| Resting Breathing Rate | < 12 breaths/min (bradypnoea): may indicate CNS depression, metabolic alkalosis, or opioid effect | AHA/ACC Vital Signs Guidelines | ◆ Moderate |
| Inspiratory-to-Expiratory (I:E) Ratio | < 1:1.5 (compressed expiratory phase): suggests dynamic hyperinflation or expiratory flow limitation | O'Donnell et al. Thorax 2001; Gibson et al. Thorax 2001 | ◆ Moderate |
| I:E Ratio | > 1:3 at rest: may indicate anxiety-driven breathing pattern or dysfunctional breathing | Courtney et al. Int J Osteopath Med 2011 | ● Informational |
| Breath-Hold Time (BOLT / first urge) | < 10 s: severe dysfunctional breathing pattern; clinical screening threshold | McKeown P, The Oxygen Advantage 2015; Barker & Everard Paediatr Respir Rev 2015 | ⚠ High Priority |
| Breath-Hold Time | < 25 s: positive screen for dysfunctional breathing (sensitivity 0.89, specificity 0.71) | Courtney et al. Int J Osteopath Med 2011 | ◆ Moderate |
| End-Tidal CO₂ Proxy (peak exhale CO₂) | < 35 mmHg equivalent at rest: suggests hyperventilation or low cardiac output state | Naifeh & Kamiya Psychophysiology 1981; Laffey & Kavanagh Br J Anaesth 2002 | ◆ Moderate |
| CO₂ Clearance Efficiency (post-breath-hold) | Slow return of exhaled CO₂ to resting baseline (> 60 s): suggests reduced ventilatory responsiveness | Dempsey et al. J Physiol 2012 | ● Informational |
| Respiratory Rate Variability (RRV) | Very high RRV (CV > 25% breath-to-breath): irregular breathing pattern; associated with dysfunctional breathing | Courtney et al. Appl Psychophysiol Biofeedback 2008 | ● Informational |
| Expiratory Flow Limitation Shape | Concave flow-volume curve on forced expiration: hallmark of obstructive disease, detectable before FEV₁/FVC falls below 0.70 | Pellegrino et al. Eur Respir J 2005 | ◆ Moderate |
| Respiratory Sinus Arrhythmia (RSA) Amplitude | Absent or very low RSA (< 2 bpm HR swing with breathing): marker of reduced vagal tone; associated with anxiety, cardiovascular disease | Grossman & Taylor Biol Psychol 2007; Thayer et al. Neurosci Biobehav Rev 2012 | ◆ Moderate |
| Cardiorespiratory Coupling (CRC) | Weak CRC at rest: diminished respiratory sinus arrhythmia; associated with autonomic dysregulation | Schulz et al. Phil Trans R Soc A 2013 | ● Informational |

A3 — Functional Mobility & Neuromuscular Flags

| Parameter | Threshold / Flag Condition | Source / Guideline | Priority |
| --- | --- | --- | --- |
| Single-Leg Balance — Eyes Open | < 10 s: highest sensitivity/specificity for fall history; strongest single fall-risk indicator in community-dwelling adults | Vellas et al. J Am Geriatr Soc 1997; Springer et al. J Geriatr Phys Ther 2007 | ⚠ High Priority |
| Single-Leg Balance — Eyes Open | < 5 s in adults under 60: warrants assessment for vestibular dysfunction, peripheral neuropathy, or CNS pathology | Bohannon et al. Arch Phys Med Rehabil 1984 | ⚠ High Priority |
| Single-Leg Balance — Left vs. Right Asymmetry | > 15% asymmetry between limbs: clinically significant; may indicate unilateral neuromuscular deficit or prior injury | Trojian & McKeag Br J Sports Med 2006 | ◆ Moderate |
| Sway Path Length (accelerometer) | High sway path length with long hold time: holds 60 s but sways continuously — neuromuscular quality flag independent of duration | Prieto et al. IEEE Trans Biomed Eng 1996 | ● Informational |
| Sway Frequency > 1 Hz | High-frequency postural tremor component: may indicate cerebellar dysfunction, essential tremor, or medication effect | Diener et al. Brain 1984; Peterka Chaos 2009 | ◆ Moderate |
| 30-Second Sit-to-Stand — Rep Count | < 8 reps (ages 60-64): below normative range; associated with elevated fall risk and functional dependence | Rikli & Jones Gerontologist 1999 | ⚠ High Priority |
| 30-Second Sit-to-Stand — Rep Count | < 14 reps (ages 60-69): below average for age group | Rikli & Jones updated norms 2013 | ◆ Moderate |
| STS Delta Time Between Reps (Fatigue Rate) | Progressive slowing > 20% from first third to final third of 30-second test: elevated neuromuscular fatigue rate | Derived from Bohannon 2006; Gill et al. J Gerontol 1995 | ● Informational |
| Timed Up and Go (if administered) | > 12 s: fall risk threshold; > 20 s: high fall risk, dependence in ADLs | Podsiadlo & Richardson J Am Geriatr Soc 1991; ATS/ERS Functional Testing Statement | ⚠ High Priority |
| Front Plank Hold Time | < 30 s in adults under 60: significantly below normative range; associated with low back pain and trunk instability | McGill SM, Low Back Disorders 2007; Schellenberg et al. J Orthop Sports Phys Ther 2007 | ◆ Moderate |
| Left-Right Plank Duration Asymmetry (side plank) | > 10% asymmetry between sides: McGill lateral endurance ratio flag for injury risk | McGill SM, Low Back Disorders 2007 | ◆ Moderate |
| Wall Sit Hold Time | < 15 s in adults under 60: severely reduced isometric quad endurance; consistent with knee osteoarthritis or significant deconditioning | Bohannon Percept Mot Skills 2006; Doherty Eur J Appl Physiol 2001 | ◆ Moderate |
| Squat Depth — Tilt Sensor | Peak hip angle < 70 degrees (thighs not reaching parallel): rep does not meet protocol standard; flagged and excluded from count | AeroCardia accelerometer/tilt protocol; Schoenfeld J Strength Cond Res 2010 | ● Informational |
| Eccentric:Concentric Ratio (squat) | E:C ratio > 2.0 (concentric phase > 2x slower than eccentric): progressive fatigue-induced motor pattern breakdown | Cormie et al. Sports Med 2011; Winter DA, Biomechanics 2009 | ● Informational |

A4 — Composite & Cross-Test Flags

These flags emerge only from comparing results across multiple tests. They are among the most clinically sensitive signals in the battery because they reflect how systems interact under load, not just resting state values.

| Parameter | Threshold / Flag Condition | Source / Guideline | Priority |
| --- | --- | --- | --- |
| SpO2 Exercise Desaturation Delta (Test 1 to Test 4) | Drop >= 3% from resting SpO2 during YMCA step: most sensitive marker for exercise-induced desaturation; flags ILD, pulmonary hypertension, severe COPD — often missed at rest | Hadeli et al. Chest 2001; Spiro et al. ATS/ERS 2003 | ⚠ High Priority |
| Ventilatory Recovery Rate (post-step test) | Breathing rate still > 20 br/min at 2 min post-exercise: slow ventilatory recovery suggests excessive anaerobic contribution or cardiovascular limitation at submaximal effort | Wasserman et al. Principles of Exercise Testing 5th ed. | ◆ Moderate |
| Cross-Test Resting HR Drift (Test 1 vs. pre-Test 4) | > 10 bpm unexplained rise: incomplete autonomic recovery or cumulative fatigue; also a data quality flag for non-compliance with inter-test rest periods | Plews et al. IJSPP 2013 | ● Informational |
| O2 Pulse Proxy Flattening (VO2/HR during step test) | O2 uptake plateaus while HR continues to rise during exercise: suggests cardiac stroke volume limitation at submaximal effort | Wasserman et al. Principles of Exercise Testing 5th ed.; Arena & Guazzi J Am Coll Cardiol 2008 | ◆ Moderate |
| Orthostatic HR Response (seated to standing at step test start) | HR rise > 30 bpm on standing: POTS screen threshold. Rise > 20 bpm in adults 60+: clinically significant orthostatic intolerance | Sheldon et al. Heart Rhythm 2015; Freeman et al. Neurology 2011 | ⚠ High Priority |
| FEV1/FVC Normal + Low SpO2 at Rest | Normal spirometry with SpO2 < 95% at rest: raises suspicion for pulmonary vascular disease (e.g. pulmonary hypertension) or intracardiac shunt — not detected by spirometry alone | Galie et al. ESC/ERS PAH Guidelines 2015 | ⚠ High Priority |
| Low HRV + Elevated Resting HR Combination | RMSSD < 20 ms combined with HR > 80 bpm: dual autonomic signal; substantially elevated cardiovascular risk vs. either marker alone | Thayer et al. Neurosci Biobehav Rev 2012; Aune et al. Eur J Prev Cardiol 2017 | ⚠ High Priority |
| Poor Balance + Poor STS + Slow HR Recovery (triple flag) | All three functional/cardiovascular scores below 25th percentile simultaneously: high-risk functional profile; strong predictor of near-term fall, hospitalisation, or loss of independence in adults 60+ | Studenski et al. JAMA 2011; Pahor et al. NEJM 2014 (LIFE Study) | ⚠ High Priority |
| HR-SpO2 Divergence During Exercise | HR rising steeply (> 20 bpm/min) while SpO2 simultaneously drops: cardiovascular system compensating for impaired gas exchange; distinct from pure deconditioning | Wasserman et al. Principles of Exercise Testing 5th ed. | ◆ Moderate |
| Breath-Hold Time < 20 s + Elevated Resting Breathing Rate > 18 br/min | Combined pattern: consistent with dysfunctional breathing syndrome (Nijmegen-positive profile) even without formal questionnaire | Thomas et al. Thorax 2001 (Nijmegen Questionnaire); Courtney et al. 2011 | ◆ Moderate |

A5 — Absolute Test Stop Conditions

The following are non-negotiable stop conditions. The app must immediately halt the active test and display a rest or seek-care prompt if any of these are detected or self-reported.

| Condition | Trigger / Threshold |
| --- | --- |
| SpO2 < 88% during any exercise test | Absolute contraindication to continued exercise — oxygen delivery critically impaired |
| HR > 90% of age-predicted maximum (220 minus age) | Test exceeds submaximal design intent; protocol is not designed for maximal exertion |
| Self-reported chest pain or pressure | Any chest discomfort during exercise — cardiac event cannot be excluded |
| Self-reported dizziness, near-syncope, or loss of balance | Immediate safety risk; also stops balance test if participant grabs support surface |
| Self-reported severe dyspnoea (unable to speak a full sentence) | Disproportionate breathlessness at submaximal load — warrants medical evaluation |
| Visible cyanosis of lips or fingernails | Clinical sign of severe hypoxaemia |
| HR drop > 10 bpm during progressive exercise (without stopping) | Unexpected exercise-induced hypotension / cardiac output failure pattern |
| Plank or wall sit: joint pain (not muscular discomfort) | Distinguishes muscular fatigue (acceptable) from joint pathology (stop condition) |

A6 — Condition Routing Flags

These are the profile conditions stored in the user's account that determine which test variant the app administers. They are not flags generated by test results — they are inputs to the routing logic. Included here for completeness as a single reference document.

| Condition Type | Routing Effect | Clinical Rationale |
| --- | --- | --- |
| Cardiac condition (any) | YMCA Step Test routed to 6-Minute Walk Test (6MWT); score label changes to Functional Exercise Capacity | YMCA step cadence is demanding for stable cardiac patients; 6MWT is the validated self-paced alternative |
| Unstable angina / MI < 1 month | All exercise tests suspended; resting and pulmonary tests only | Absolute contraindication to exercise testing per AHA/ACC guidelines |
| Mobility-limiting condition (knee, hip, ankle pathology) | Squat routed to 30-second STS; balance remains eyes-open only | Full squat depth not safely achievable; dynamic STS is lower-load alternative |
| Age > 45, no other condition | Balance: eyes-open only (standard); Strength: 30-second STS | Eyes-open balance discriminates well in 46+ population; STS norms well-validated for this age group |
| Age <= 45, no limiting conditions | Balance: eyes-open + eyes-closed composite; Strength: squat to exhaustion | Eyes-open balance has severe ceiling effect (mean ~45 s vs. 60 s cap); squat provides better discrimination |
| Respiratory / pulmonary condition | No routing change; all respiratory metrics flagged for enhanced review | All pulmonary tests remain; results interpreted with heightened attention to FEV1/FVC, ETCO2 proxy, SpO2 delta |
| History of falls (self-reported) | Balance: add eyes-closed condition regardless of age; alert clinician reviewer | Fall history is the strongest predictor of future falls; enhanced balance assessment warranted |

| APP STATE FLAGS Test 8 · Wall Sit |  |  |
| --- | --- | --- |
| Phase | State Code Emitted | What it marks / what to extract |
| Position check | T8.POSCHECK | Tilt sensor verifies knee angle 85-95 deg. App prompts adjustment if outside range. Locked angle recorded. |
| Hold start | T8.HOLD.START | Hold clock starts. HR tracking active (isometric HR elevation = secondary CV signal). Tilt monitoring for form break. |
| Arm flag | T8.ARM.FLAG | Accel detects hands pressing on thighs. Protocol deviation logged — test continues but flagged in results. |
| Form break | T8.FORM.BREAK | Knee angle rises >15 deg from locked position. Hold ends if not self-corrected within 3 s. |
| Cap reached | T8.CAP | 2-min cap reached. Duration = 120 s. |
| Hold end | T8.HOLD.END | Duration and HR-during-hold locked. Isometric HR elevation vs. T1 baseline computed. |
| Session end | T8.SESSION.COMPLETE | 30-min session closed. All domain scores generated. Full report output triggered. |

Appendix C — Continuous Capture Opportunities

The AeroCardia sensor stack emits data continuously throughout the session, not only at the phase-boundary timestamps defined in the state flag tables. This appendix identifies where capturing the full time-series signal — rather than extracting only the snapshot values already defined — meaningfully increases diagnostic and research utility. Each opportunity is annotated with the sensor stream, the window boundaries (using the state codes from Appendix B), the derived metric or feature that becomes accessible, and the reason a snapshot alone misses it.

C1 — Why Continuums Change What You Can Measure

A snapshot captures a value at a moment. A continuum captures how a value changes — its trajectory, rate, variability, and shape. For physiological signals, the trajectory is often more clinically informative than any single point. Three categories of benefit apply across the AeroCardia battery:

| Snapshot gives you... | Continuum adds... | Clinical value |
| --- | --- | --- |
| A value at one moment | Rate of change and trajectory | Distinguishes stable compensation from active decompensation; detects trends invisible at any single point |
| A peak or nadir | The shape of rise and fall | Kinetics of SpO2 drop and recovery, CO2 clearance half-life, HR ramp slope — all require the full curve |
| Whether a threshold was crossed | When, how fast, and whether it recovered | Physiological resilience: a system that crosses a threshold but recovers quickly differs fundamentally from one that stays there |
| A mean or average | Variability and rhythmic structure | HRV, RRV, sway frequency, cadence consistency — all are variability metrics that collapse to zero under averaging |

C2 — Per-Test Continuum Capture Opportunities

The table below lists every sensor stream where continuous capture adds measurable benefit beyond the snapshot values already defined. Window boundaries are given as state code pairs. Priority reflects both the clinical value of the continuum metric and the readiness of normative data to interpret it.

| Test | Sensor Stream | Window / (start → end) | Continuum Metric / Feature | Why a snapshot misses it | Priority |
| --- | --- | --- | --- | --- | --- |
| Test 1 · Resting Baseline |  |  |  |  |  |
| T1 | HR | T1.QUIET.START → T1.QUIET.END | HRV / RMSSD, pNN50, LF/HF power spectral decomposition | HRV requires the full R-R interval sequence (~120 s minimum). A single HR snapshot gives rate only — no variability, no autonomic balance. | ⚠ High |
| T1 | HR | T1.QUIET.START → T1.QUIET.END | Respiratory Sinus Arrhythmia (RSA) amplitude | RSA is the rhythmic HR fluctuation locked to breathing. Requires concurrent HR + breathing rate time series to isolate the respiratory-frequency component of HR. | ⚠ High |
| T1 | Pressure (exhale/inhale) | T1.QUIET.START → T1.QUIET.END | Breathing rate variability (RRV) and I:E ratio per breath | A mean I:E ratio hides breath-to-breath irregularity. High RRV (CV >25%) is a dysfunctional breathing flag that collapses to zero under averaging. | ◆ Mod |
| T1 | CO2 (exhaled) | T1.QUIET.START → T1.QUIET.END | Breath-to-breath ETCO2 trend and stability | A single end-tidal CO2 value doesn't reveal whether the patient is hyperventilating intermittently or stably. Trend identifies compensatory hyperventilation. | ◆ Mod |
| T1 | O2 | T1.QUIET.START → T1.QUIET.END | O2 consumption trend and metabolic stability at rest | Falling resting O2 uptake over the 2-min window indicates the patient hasn't reached true metabolic rest — snapshot would overestimate resting VO2. | ◆ Mod |
| T1 | Accelerometer | T1.QUIET.START → T1.QUIET.END | Resting movement artefact / trunk sway at seated rest | Continuous accel stream detects if participant is shifting, fidgeting, or has a postural tremor — which confounds all T1 resting values if undetected. | ◆ Mod |
| T1 | HR + Pressure | T1.DEEP.START → T1.DEEP.END | Cardiorespiratory coupling (CRC) during deep breathing | CRC requires the concurrent phase relationship between HR oscillations and breathing pressure over multiple breath cycles. Impossible to compute from snapshots. | ● Research |
| Test 2 · Spirometry |  |  |  |  |  |
| T2 | Pressure (exhale) | T2.TRIAL.START[n] → T2.EXHALE.END[n] | Full flow-volume curve shape per trial | FEV1 and FVC are two points on the flow-volume curve. The full curve shape reveals expiratory flow limitation, concavity (obstruction pattern), and effort quality that FEV1/FVC ratio alone cannot capture. | ⚠ High |
| T2 | Pressure | T2.TRIAL.START[n] → T2.EXHALE.END[n] | FEV0.5 and mid-expiratory flow (FEF25-75%) | FEF25-75% requires integrating flow across the middle half of the forced expiration. Requires continuous pressure signal — not derivable from endpoints alone. | ⚠ High |
| T2 | Pressure | T2.TRIAL.START[n] → T2.EXHALE.END[n] | Effort quality index — peak flow timing and plateau detection | A technically poor effort (hesitant start, early termination, cough) is only detectable from the pressure trace shape. Spotting poor effort prevents false low FEV1 from being scored. | ⚠ High |
| T2 | HR | T2.TRIAL.START[n] → T2.EXHALE.END[n] | Valsalva-induced HR response pattern | Forced expiration briefly raises intrathoracic pressure, producing a characteristic HR dip. Continuous HR during the effort reveals autonomic responsiveness as a free bonus signal. | ● Research |
| T2 | O2 + CO2 | T2.INTERTRIAL[n] (60-s rest) | Gas washout / ventilatory settling rate between trials | Rate at which O2 and CO2 return to inter-trial baseline reveals ventilatory efficiency. Slow washout between trials suggests incomplete recovery — affects trial comparison validity. | ● Research |
| Test 3 · YMCA Step Test |  |  |  |  |  |
| T3 | HR | T3.STANDING → T3.STEP.END | HR kinetics: ramp rate, linearity with load, plateau detection | HR at 1-min and 2-min snapshots describe only two points. The full HR ramp reveals whether the response is linear (normal), supra-linear (deconditioning), or shows a premature plateau (chronotropic incompetence). | ⚠ High |
| T3 | SpO2 | T3.STEP.START → T3.REC.2MIN | SpO2 trajectory: nadir timing, recovery slope, exercise + recovery kinetics | The SpO2 delta flag checks whether a threshold is crossed. The continuous curve reveals when the nadir occurs (early = ventilatory limitation, late = perfusion lag), how steep the drop is, and whether recovery is exponential or prolonged. | ⚠ High |
| T3 | CO2 (exhaled) | T3.STEP.START → T3.VENT.SETTLED | VE/VCO2 slope across the step test | VE/VCO2 slope requires breath-by-breath CO2 output plotted against ventilation across increasing workload. Two snapshot CO2 values cannot reconstruct this. The slope is the most sensitive heart-failure marker in the battery. | ⚠ High |
| T3 | O2 | T3.STEP.START → T3.STEP.END | O2 pulse proxy (O2 uptake / HR) at each breath | O2 pulse flattening during exercise — a key signal for stroke volume limitation — requires the continuous O2 and HR streams to track their ratio as exercise progresses. A terminal snapshot misses the flattening. | ⚠ High |
| T3 | Pressure / breathing rate | T3.STEP.START → T3.VENT.SETTLED | Ventilatory equivalent (VE/VO2) trajectory and RCP detection | The respiratory compensation point (RCP) is identified when VE/VO2 begins to rise without a concurrent rise in VE/VCO2. Requires the full continuous ventilation trace — invisible from snapshots. | ◆ Mod |
| T3 | HR | T3.STEP.END → T3.REC.2MIN | HR recovery kinetics: first-30-second slope and time constant | 1-min recovery HR is one point. The full recovery curve provides the HR decay time constant, which is more sensitive to autonomic dysfunction than any single recovery snapshot. | ⚠ High |
| T3 | Accelerometer | T3.STEP.START → T3.STEP.END | Step cadence accuracy and left-right symmetry per stride | Cadence at 96 bpm is the protocol standard. Continuous accel detects drift (fatigue-related slowdown), asymmetry between legs, and whether the protocol was followed — essential data quality checks. | ◆ Mod |
| T3 | SpO2 + HR | T3.REC.1MIN → T3.REC.2MIN | HR-SpO2 divergence: HR recovering while SpO2 stays depressed | If HR recovers normally but SpO2 remains low, the limitation is ventilatory/gas exchange rather than cardiac. This distinction requires concurrent continuous streams, not isolated snapshots. | ⚠ High |
| Test 4 · Single-Leg Balance |  |  |  |  |  |
| T4 | Accelerometer | T4.R.EO.START → T4.R.EO.END (and L equivalent) | Postural sway path length (total AP + ML displacement) | Hold duration measures how long balance was maintained. Sway path length measures the neuromuscular effort required — a participant who holds 60 s with constant large sways has fundamentally different balance quality than one who holds 60 s with minimal sway. | ⚠ High |
| T4 | Accelerometer | T4.R.EO.START → T4.R.EO.END | Sway frequency decomposition (low-freq vestibular vs. high-freq neuromuscular vs. tremor band >1 Hz) | Different frequency bands in sway correspond to different physiological systems. Cannot be distinguished without the continuous time-series — requires FFT or wavelet decomposition. | ◆ Mod |
| T4 | Accelerometer | T4.R.EO.START → T4.R.EO.END | Sway velocity and direction bias (anterior-posterior vs. mediolateral dominance) | Snapshot gives duration. Velocity and direction bias localise the neuromuscular deficit — mediolateral sway indicates hip abductor weakness; AP sway indicates ankle strategy deficits. | ◆ Mod |
| T4 | Accelerometer | T4.R.EO.START → T4.L.EC.END | Sway entropy / complexity (approximate entropy of accel trace) | Healthy balance control is complex and irregular, not simply quiet. Low entropy (overly regular sway) paradoxically indicates reduced adaptability and is associated with fall risk independently of sway magnitude. | ● Research |
| T4 | HR | T4.R.EO.START → T4.L.EC.END | HR elevation and variability during balance challenge | Balance requires mild cardiovascular effort. Continuous HR during the hold detects whether a participant's cardiovascular system is unusually taxed by the postural challenge — a secondary autonomic signal. | ● Research |
| Test 5 · Lower-Body Strength |  |  |  |  |  |
| T5 | Accelerometer | T5.START → T5.END | Rep-to-rep delta time (fatigue kinetics curve) | The fatigue flag checks the final-third average against the first-third average. Continuous timestamps for each rep give the full fatigue curve shape — exponential (muscle fatigue) vs. linear (pacing) vs. step-change (strategy shift). | ⚠ High |
| T5 | Accelerometer | T5.START → T5.END | Eccentric:concentric ratio per rep (time spent lowering vs. rising) | E:C ratio captures motor control breakdown — when the eccentric phase slows disproportionately, the participant is losing neuromuscular control. Requires timestamps of each phase within each rep. | ◆ Mod |
| T5 | Tilt (angular velocity) | T5.START → T5.END | Angular velocity of descent per rep — power output proxy | Rep count is a count. Angular velocity gives the speed of movement, which is a proxy for lower-body power. Power declines faster than count as fatigue sets in. | ◆ Mod |
| T5 | HR | T5.START → T5.END | HR ramp during STS: metabolic demand of repeated sit-to-stand | For the 30-second STS, continuous HR shows metabolic cost per rep and whether HR reaches a plateau (cardiovascular limiting) or keeps climbing (purely muscular). Free signal given HR is already active. | ◆ Mod |
| T5 | SpO2 | T5.START → T5.END | SpO2 during lower-body exertion: local vs. systemic oxygen demand | SpO2 drop during STS/squat is less common than during the step test but, if it occurs, is diagnostically more significant — suggests inadequate cardiac output for even modest lower-body exertion. | ◆ Mod |
| Test 7 · Front Plank |  |  |  |  |  |
| T7 | Tilt / Accelerometer | T7.HOLD.START → T7.HOLD.END | Hip angle drift curve and sag onset kinetics | The sag fail flag is binary. Continuous tilt gives the rate of hip angle drift — fast drift indicates rapid fatigue failure; slow drift reveals how long true near-failure endurance was maintained before the threshold. | ◆ Mod |
| T7 | Accelerometer | T7.HOLD.START → T7.HOLD.END | Trunk micro-oscillation (Pilates-stability tremor) frequency | During extended holds, the trunk exhibits low-amplitude oscillations. Frequency and amplitude of this tremor are a finer-grained measure of core endurance than binary pass/fail. | ● Research |
| T7 | HR | T7.HOLD.START → T7.HOLD.END | HR elevation kinetics during isometric hold | Isometric contractions produce a characteristic HR and BP elevation (the pressor response). Rate of HR rise during the plank is a measure of isometric cardiovascular stress — more sensitive than hold time alone. | ● Research |
| Test 8 · Wall Sit |  |  |  |  |  |
| T8 | Tilt | T8.HOLD.START → T8.HOLD.END | Knee angle drift curve: rate of position loss as quads fatigue | Form break ends the test at >15 deg deviation. Continuous tilt shows the rate of angle drift before that threshold, giving a richer picture of quad endurance capacity. | ◆ Mod |
| T8 | HR | T8.HOLD.START → T8.HOLD.END | Isometric pressor response: HR trajectory during sustained quad contraction | The isometric pressor response (HR and BP rise) scales with muscle mass engaged and contraction intensity. HR trajectory during the wall sit adds a cardiovascular load dimension to what is otherwise a muscular endurance test. | ◆ Mod |
| T8 | HR | T7.HOLD.START → T8.HOLD.END (Tests 7+8 combined) | Cumulative isometric cardiovascular load: back-to-back isometric test HR recovery | Tests 7 and 8 are consecutive isometric holds. HR recovery between them (T7.COMPLETE → T8.HOLD.START) reveals whether the cardiovascular system recovered or carried forward fatigue — a session-level stress index. | ◆ Mod |

C3 — Session-Level Continuums (Cross-Test)

Some of the most valuable continuous signals span the entire session rather than being bounded by a single test. These require the data pipeline to maintain an unbroken time-series across all state code boundaries.

| Stream | Session Window | Session-Level Metric and Value |
| --- | --- | --- |
| HR | T1.QUIET.START → T8.SESSION.COMPLETE | Session HR drift: baseline HR at T1 vs. pre-test HR readings before each subsequent test. A progressive rise across tests (>10 bpm from T1 to T5) indicates cumulative cardiovascular stress or incomplete inter-test recovery — a data quality and physiological safety signal. |
| SpO2 | T1.QUIET.START → T8.SESSION.COMPLETE | Session SpO2 floor: the lowest SpO2 value across all tests and all phases. If this occurs at rest or during low-exertion tests rather than at the YMCA step test peak, the clinical significance is substantially different. |
| HR | T1.QUIET.START → T5.COMPLETE | Inter-test HRV trajectory: HRV computed from the inter-test rest windows between each test. Declining HRV across the session is a sensitive marker of cumulative autonomic load — more sensitive than any per-test HRV snapshot. |
| CO2 (exhaled) | T1.QUIET.START → T5.COMPLETE | Resting ETCO2 trend across inter-test rests: a participant who is gradually hyperventilating between tests (falling inter-test ETCO2) may be developing anxiety or dysfunctional breathing in response to test demands — contaminates all CO2-dependent metrics if undetected. |
| Accelerometer | T1.QUIET.START → T8.SESSION.COMPLETE | Movement artefact log: a continuous record of accelerometer RMS during nominally stationary phases (pre-rests, inter-test rests, seated recovery). Used to flag sessions where participant movement is contaminating resting-state metrics. |
| HR + SpO2 | T3.STEP.START → T3.VENT.SETTLED | Cardiorespiratory response surface during the step test: the joint trajectory of HR and SpO2 defines four quadrants (HR up/SpO2 stable = normal cardiovascular response; HR up/SpO2 down = gas exchange limitation; HR flat/SpO2 stable = chronotropic incompetence; HR flat/SpO2 down = critical limitation). Requires both continuous streams simultaneously. |

C4 — Derived Metrics Only Accessible via Continuous Capture

The following metrics are structurally impossible to derive from snapshots. They are listed here as a reference for the data pipeline — if the raw streams are stored continuously, these can be computed retrospectively without re-testing.

| Metric | Required streams | Computation requires... |
| --- | --- | --- |
| HRV / RMSSD, pNN50, LF/HF | HR (continuous R-R interval) | Minimum ~120 consecutive R-R intervals. Computed from T1.QUIET.START → T1.QUIET.END. Impossible from any HR snapshot. |
| Respiratory Sinus Arrhythmia (RSA) | HR + breathing rate (concurrent) | Cross-spectrum of HR and breathing rate time series. Requires the phase relationship between them — collapses to zero if either stream is sampled. |
| Cardiorespiratory Coupling (CRC) | HR + pressure (concurrent) | Mutual information or phase synchrony between HR oscillations and breathing pressure cycles over >= 10 breath cycles. |
| VE/VCO2 slope | CO2 + pressure (breath-by-breath) | Linear regression of ventilation vs. CO2 output across the step test. Requires >= 20 breath-by-breath data points from T3.STEP.START onwards. |
| O2 pulse trajectory and flattening | O2 + HR (concurrent) | Ratio O2 uptake / HR tracked over time during exercise. Flattening of this ratio = stroke volume plateau. Requires both streams from T3.STEP.START → T3.STEP.END. |
| HR recovery time constant (tau) | HR (continuous post-exercise) | Exponential fit to the HR decay curve from T3.STEP.END → T3.REC.2MIN. More sensitive than the 1-min recovery snapshot. Requires >= 40 data points across recovery. |
| Flow-volume curve shape / FEF25-75% | Pressure (continuous during FVC effort) | Integration of flow across the middle 50% of forced expiration. Requires the full pressure trace from T2.TRIAL.START[n] → T2.EXHALE.END[n] at adequate sampling rate (>= 100 Hz). |
| Sway path length and frequency decomposition | Accelerometer (3-axis, continuous) | FFT or wavelet decomposition of the AP and ML accel traces during T4 hold windows. Requires uninterrupted signal at >= 50 Hz sampling rate. |
| Fatigue kinetics curve (delta-time per rep) | Accelerometer (timestamp per rep event) | Inter-rep interval time series from T5.REP[1] → T5.REP[n]. Requires per-rep timestamps, not just a final count. |
| SpO2 nadir timing and recovery slope | SpO2 (continuous) | Time from T3.STEP.END to SpO2 nadir, and slope of recovery from nadir to T3.REC.2MIN. Localises whether gas exchange limitation is ventilatory (early nadir) or perfusion-limited (delayed nadir). |
| Isometric pressor response slope | HR (continuous during T7 and T8) | Rate of HR rise per second from T7.HOLD.START (plank) and T8.HOLD.START (wall sit). Scales with isometric cardiovascular load. Requires >= 30 s of continuous HR during each hold. |
| CO2 clearance half-life (post breath-hold) | CO2 (continuous post-exhale) | Exponential fit to CO2 return towards baseline from T3.EXHALE → T3.CO2BASELINE. Characterises ventilatory responsiveness to hypercapnia. Requires breath-by-breath CO2 from T3.EXHALE for minimum 60 s. |

C5 — Data Labelling Guide: Mapping Continuums to State Codes

For the data pipeline and ML training workflows, every continuous segment should be tagged with its enclosing state code pair as a label. The following conventions ensure that continuum windows are unambiguously defined and reproducible across sessions.

• A continuum window is defined as the half-open interval [START_CODE, END_CODE) — the sample at the START code timestamp is included; the sample at the END code timestamp is excluded. This ensures no sample belongs to two windows.

• If an END code is not emitted (e.g. participant stops early, safety stop condition triggered), the window closes at the STOP event timestamp. All metrics derived from such truncated windows should be flagged as INCOMPLETE and excluded from normative scoring but retained for research.

• For per-rep metrics (T5.REP[n], T2.TRIAL.START[n] etc.), the index n is 1-based. The window for rep n is [T5.REP[n-1], T5.REP[n]) where rep 0 is defined as T5.START. The delta-time for rep n is therefore T5.REP[n].timestamp - T5.REP[n-1].timestamp.

• The eyes-closed balance windows (T4.R.EC.START / T4.L.EC.START) are only emitted for participants aged 45 or under. Data pipelines must check for the presence of these codes before attempting to extract EC balance metrics — absence means the age-routing excluded this variant, not a recording failure.

• Session-level continuums (C3) should be labelled with the enclosing session ID and the state code pair from the first START to the last END code in the session. They are distinct label classes from per-test windows and should not be merged with them in training datasets.

• Recommended minimum sampling rates for each stream to support all continuum metrics in C4: HR >= 1 Hz (prefer R-R interval, ~2-5 Hz effective); SpO2 >= 1 Hz; O2/CO2 >= 1 Hz (breath-by-breath, effectively 0.25-0.5 Hz at rest); Pressure >= 100 Hz (spirometry); Accelerometer >= 50 Hz; Tilt >= 20 Hz.